Blend Color Checker - Detailed Documentation

(For FlexChroma™ Blend Filament Series)

1. Introduction

The FlexChroma™ Blend Color Checker is a dedicated evaluation model designed to visually assess how each blend ratio of FlexChroma™ Blend Filament appears in real prints.
By using a blended filament divided into 21 sequential blend-ratio segments, generated through AMS, this tool allows you to directly observe how numerical blend ratios correspond to actual printed colors.

2. Purpose of the Color Checker

This model is optimized for the following use cases:

- Visually checking actual color differences at each blend ratio

- Observing color variation caused by print-direction changes

- Enabling numerical color evaluation using the NIX MINI 3 color-measurement tool

3. About the Included Data

3.1 Check Filament Data (Two Types)

Two filament data sets are provided, matching the two categories used in Blend Filament creation.
If you already have an approximate target blend ratio in mind, choose the data set that includes that ratio.
The blend specifications for 5%, 10%, 90%, and 95% are identical to those used in the Blend Filament series.

■ Standard Version

Actual blend-ratio structure (21 stages total):

0, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65, 70, 75, 80, 85, 90, 95, 100%

■ Detail Version

Actual blend-ratio structure:

- Low-ratio zone (10 stages):  0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10%

- High-ratio zone (10 stages):  90, 91, 92, 93, 94, 95, 96, 97, 98, 99, 100%

- One additional segment connects 10% to 90% continuously in the center.

3.2 Check Tower (1 Model)

This vertical tower is printed using the Check Filament and makes each blend step easy to identify.

- Notches are placed at intervals that match the blend-ratio switching heights.

- Larger notches appear at 20%, 25%, 45%, 50%, 55%, 75%, 80% as four key marker positions.

- Because the printed color can vary depending on the toolhead’s moving direction, the tower includes four measurable faces, allowing you to check directional color differences.

4. Recommended Workflow

- Select two filament colors that appear capable of producing the desired final color.

- Print the Check Filament using those two colors.

- Print the Check Tower using the Check Filament and identify the blend ratio closest to your target color.

- If needed, print the S4 Sample Model using the chosen ratio for a near-final test.

- Once confirmed, generate the required amount of Blend Filament using that ratio.

5. Notes for Printing

5.1 When Printing the Check Filament

- AMS is required

- 0.4 mm nozzle is required

- Recommended build plate: Textured PEI

- Do not mix incompatible materials
(Combinations such as PLA Basic + PLA Matte may often work, but this is at your own risk.)

- Glue should not be used

- Clean both the build plate and your hands before printing

5.2 When Printing the Check Tower

- Remove the brim from the Check Filament before use.

- The side marked “IN” on the brim must face the loading direction.

- Remove any burrs or debris with nippers or a knife.

- Load the Check Filament from an external spool, not through AMS.

AMS may perform unexpected purge operations, even during single-color printing.
These purges can shorten the intended segment lengths, which reduces accuracy when evaluating the Color Checker.
External-spool loading prevents these purges and preserves the exact segment structure.

- Because startup purge amounts and line-purge amounts are not constant,
the actual blend-switch positions may not perfectly match the tower’s notch positions.

6. External Measurement Tools

The Blend Color Checker is optimized for NIX MINI 3, enabling accurate numerical color evaluation.

• Optimized Segment Width

Each segment is designed with a width that provides sufficient measurement area for NIX MINI 3.
The notch width is matched to the measurement edge, helping prevent alignment errors.

• Optimized Tower Width

The tower width fully covers NIX MINI 3’s sensor area, preventing measurement failures.

• Markings on the Top Surface

The top surface includes alphabet markers corresponding to each of the four faces.
When recording results from multiple sides, these letters help you track which face was measured.

7. Summary

The Blend Color Checker is a highly reliable tool for accurately assessing
how blend ratios correspond to actual printed colors in the FlexChroma™ Blend Filament system.

With:

- Both Standard and Detail Check Filaments

- External-spool loading to maintain stable segment boundaries

- A structure optimized for NIX MINI 3

you can confidently select the blend ratio that best suits your project.